############################################################

# EU4Green – Landscape connectivity analysis with grainscape

# Annotated script

# December 2025

# Martin Suanjak, Phlipp Semenchuk & Katharina Huchler

# Environment Agency Austria https://www.umweltbundesamt.at/

############################################################

############################

# Load required libraries

############################

library(grainscape)

library(terra)

library(raster)

library(tidyverse)

library(igraph)

library(sf)

library(RColorBrewer)

library(igraph)

library(tidytext)

library(knitr)

library(ggnewscale)

############################

# Load CORINE land cover raster

############################

corine <- rast("path to .tif file")

############################

# Load study area polygons

############################

study_a <- vect("path to .shp file of target area") |>

project("epsg:3035")

plot(study_a)

#choose one polygon if there are several in the file

study_a <- study_a[1]

############################

# Create buffer around study area

############################

# 25 km buffer to include surrounding landscape context

buffered_area <- buffer(study_a, width = 25000)

plot(buffered_area, border = "red", lty = 2)  # Dashed red outline

plot(study_a, border = "black", col = NA, add = TRUE)

corine_clipped <- crop(corine, buffered_area)

corine_clipped <- mask(corine_clipped, buffered_area)

plot(corine_clipped)

plot(study_a, border = "black", col = NA, add = TRUE)

plot(buffered_area, border = "red", lty = 2, add=TRUE)

############################################################

# Create resistance layer – example: forest species

############################################################

# Resistance values represent movement difficulty

# Lower values = more permeable habitat

# choose Broad-leaved forest or Natural grasslands

{

feature <- "Broad-leaved forest"

corine_feature <- corine_clipped

corine_feature$feature <- corine_feature$LABEL3

corine_feature$feature <- ifel(corine_feature$feature == feature , 1 , NA)

corine_feature$resistance <- NA

corine_feature$resistance <- ifel(corine_feature$LABEL3 == feature , 1 , corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Broad-leaved forest", 1, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Mixed forest", 1, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Land principally occupied by agriculture, with significant areas of natural vegetation", 2, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Agro-forestry areas", 2, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Sclerophyllous vegetation", 2, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Coniferous forest", 2, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Moors and heathland", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Pastures", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Peat bogs", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Road and rail networks and associated land", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Green urban areas", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Olive groves", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Complex cultivation patterns", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Transitional woodland-shrub", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Burnt areas", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Fruit trees and berry plantations", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Annual crops associated with permanent crops", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Natural grasslands", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Sparsely vegetated areas", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Airports", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Non-irrigated arable land", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Permanently irrigated land", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Rice fields", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Vineyards", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Beaches, dunes, sands", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Bare rocks", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Inland marshes", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Salt marshes", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Continuous urban fabric", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Discontinuous urban fabric", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Industrial or commercial units", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Port areas", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Mineral extraction sites", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Dump sites", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Construction sites", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Sport and leisure facilities", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Glaciers and perpetual snow", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Salines", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Intertidal flats", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Water courses", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Water bodies", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Coastal lagoons", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Estuaries", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Sea and ocean", 8, corine_feature$resistance)

}

#or choose Natural grasslands

{

feature <- "Natural grasslands"

corine_feature <- corine_clipped

corine_feature$feature <- corine_feature$LABEL3

corine_feature$feature <- ifel(corine_feature$feature == feature , 1 , NA)

corine_feature$resistance <- NA

corine_feature$resistance <- ifel(corine_feature$LABEL3 == feature , 1 , corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Moors and heathland", 1, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Natural grasslands", 1, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Land principally occupied by agriculture, with significant areas of natural vegetation", 2, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Pastures", 2, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Peat bogs", 2, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Sparsely vegetated areas", 2, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Agro-forestry areas", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Sclerophyllous vegetation", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Road and rail networks and associated land", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Green urban areas", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Olive groves", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Complex cultivation patterns", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Transitional woodland-shrub", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Burnt areas", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Airports", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Non-irrigated arable land", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Permanently irrigated land", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Rice fields", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Vineyards", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Beaches, dunes, sands", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Bare rocks", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Inland marshes", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Salt marshes", 4, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Broad-leaved forest", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Mixed forest", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Coniferous forest", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Fruit trees and berry plantations", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Annual crops associated with permanent crops", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Continuous urban fabric", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Discontinuous urban fabric", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Industrial or commercial units", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Port areas", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Mineral extraction sites", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Dump sites", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Construction sites", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Sport and leisure facilities", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Glaciers and perpetual snow", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Salines", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Intertidal flats", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Water courses", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Water bodies", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Coastal lagoons", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Estuaries", 8, corine_feature$resistance)

corine_feature$resistance <- ifel(corine_feature$LABEL3 == "Sea and ocean", 8, corine_feature$resistance)

}

############################################################

# Identify patches of target feature

############################################################

{

#plot restistance layer

plot(corine_feature$resistance)

plot(study_a, border = "black", col = NA, add = TRUE)

plot(buffered_area, border = "red", lty = 2, add=TRUE)

#plot target feature

corine_single <- corine_feature$feature

plot(corine_single)

plot(study_a, border = "black", col = NA, add = TRUE)

plot(buffered_area, border = "red", lty = 2, add=TRUE)

clumped <- patches(corine_single, directions = 8)

# Count unique patch IDs (excluding NA)

unique_patches <- unique(values(clumped))

unique_patches <- unique_patches[!is.na(unique_patches)]  # Remove NA if present

num_patches <- length(unique_patches)  # Get the number of patches

# Generate random colors

set.seed(42)  # Set seed for reproducibility (optional)

patch_colors <- sample(colors(), num_patches)  # Random colors

# Plot with random colors

plot(clumped, col = patch_colors, main = "Identified Patches of feature", legend = FALSE)

plot(study_a, border = "black", col = NA, add = TRUE)

plot(buffered_area, border = "red", lty = 2, add=TRUE)

patch_sizes <- as.data.frame(freq(clumped))

colnames(patch_sizes) <- c("layer", "patch_id", "pixel_count")

head(patch_sizes)

patch_sizes_sorted <- patch_sizes[order(-patch_sizes$pixel_count), ]

patch_sizes_sorted

patches <- patch_sizes_sorted[1:10, ] #hier einfach die 10 größtes Patches ausgewählt, es kann aber auch nach einer bestimmen Größe sein.

#patches

#patches <- patch_sizes_sorted[patch_sizes_sorted$pixel_count >= pix_targ, ]

class(clumped)

top10_patch_ids <- patches$patch_id

large_patches <- ifel(clumped == top10_patch_ids, clumped, NA)

plot(large_patches$lyr1)

merged_patches <- app(large_patches, fun = sum, na.rm = TRUE)

set.seed(42)

num_patches <- length(unique(values(merged_patches)))  # Count unique patch values

patch_colors <- sample(rainbow(num_patches))  # Generate distinct colors

# Plot everything in ONE figure with different colors

plot(merged_patches, col = patch_colors, main = "Top 10 largest patches", legend = FALSE)

plot(study_a, border = "black", col = NA, add = TRUE)

plot(buffered_area, border = "red", lty = 2, add=TRUE)

sink_patches <- merged_patches

}

###!!! This has only to be done when one of the largest patches does not connect with the target area

{

# Mask focal habitat to study area

corine_single <- mask(corine_feature$feature, study_a)

# Identify habitat patches (8-neighbour connectivity)

clumped <- patches(corine_single, directions = 8)

# Calculate patch sizes

patch_sizes <- as.data.frame(freq(clumped))

colnames(patch_sizes) <- c("layer", "patch_id", "pixel_count")

# Select largest patch (can be replaced by size threshold)

largest_patch_id <- patch_sizes$patch_id[which.max(patch_sizes$pixel_count)]

# Extract largest patch

source_patch <- ifel(clumped == largest_patch_id, clumped, NA)

# Combine source patch with existing sink patches

sink_patches <- merge(sink_patches, source_patch)

# Optional quick check plot

plot(sink_patches)

plot(study_a, border = "black", col = NA, add = TRUE)

plot(buffered_area, border = "red", lty = 2, add = TRUE)

}

######

############################################################

# Convert patches to binary raster

############################################################

{

merged_patches <- sink_patches

plot(merged_patches)

str(merged_patches)

binary_raster <- classify(merged_patches, cbind(NA, 0))  # First turn NA into 0

binary_raster[binary_raster != 0] <- 1                   # Then set all non-zero to 1

# Plot the result

plot(binary_raster)

patches <- binary_raster

}

######

############################################################

# Minimal Planar Graph (MPG) connectivity

############################################################

{

patchyCost <- raster(corine_feature$resistance)

plot(patchyCost)

patches <- raster(patches)

plot(patches)

patchyMPG <- MPG(patchyCost, patches)

mpg <- MPG(patchyCost, patch = patches)

mpgPlot <- mpg@mpgPlot  # Extract the mpgPlot from the mpg object

mpgPlot_df <- as.data.frame(mpgPlot, xy = TRUE, na.rm = TRUE)

# Plot using ggplot2

mpgLinks <- mpg@lcpPerimWeight

head(mpgLinks)

mpgPlot_df <- as.data.frame(mpgLinks, xy = TRUE, na.rm = TRUE)

head(mpgPlot_df)

ggplot(mpgPlot_df, aes(x = x, y = y, fill = resistance)) +

geom_raster() +

scale_fill_viridis_c() +

coord_equal() +

theme_minimal() +

ggtitle("MPG Plot (RasterLayer Version)")

#make corridors between patches thicker for better visibility

# 1. Create a RasterLayer from your data frame

r <- rasterFromXYZ(mpgPlot_df)

# 2. Create a focal function to expand resistance values

# This keeps the minimum resistance value in neighborhood

expand_resistance <- function(x) {

center <- x[ceiling(length(x)/2)]

if (!is.na(center)) {

return(center)  # keep original

} else {

# Return the minimum resistance from neighbors, ignoring NA

neighbors <- x[!is.na(x)]

if (length(neighbors) > 0) {

return(min(neighbors))

} else {

return(NA)

}

}

}

# 3. Define a larger focal window (e.g. 13x13 for ~6 pixel radius)

w <- matrix(1, nrow = 7, ncol = 7)

# 4. Apply focal to expand paths

expanded_r <- focal(r, w, fun = expand_resistance, pad = TRUE, padValue = NA)

# 5. Plot to verify

plot(expanded_r, main = "Expanded Paths with Resistance Values")

expanded_r_df <- as.data.frame(expanded_r, xy = TRUE, na.rm = TRUE)

# Plot using ggplot2

ggplot(expanded_r_df, aes(x = x, y = y, fill = layer)) +

geom_tile() +  # Use geom_tile to plot the raster as tiles

scale_fill_viridis_c() +  # Use a color scale for resistance values

coord_equal() +  # Maintain equal aspect ratio

theme_minimal() +  # Clean theme

ggtitle("Expanded Paths with Resistance Values")

merged_patches_df <- as.data.frame(merged_patches, xy = TRUE)

study_a_sf <- st_as_sf(study_a)

buffered_area_sf <- st_as_sf(buffered_area)

#cost_masked <- mask(patchyCost, buffered_area)

patchyCost <- rast(patchyCost)

cost_df <- as.data.frame(patchyCost, xy = TRUE)

names(cost_df)[3] <- "resistance"

}

#final plot

feature

area <- "Insert name of area"

g1 <-ggplot() +

geom_raster(data = cost_df, aes(x = x, y = y, fill = resistance), alpha = 0.6) +

scale_fill_gradient(name = "resistance of landscape", low = "white", high = "black") +

new_scale_fill() +

geom_tile(data = expanded_r_df, aes(x = x, y = y, fill = layer)) +

scale_fill_gradient(name = "cost of corridor", low = "forestgreen", high = "red") +

geom_tile(data = merged_patches_df, aes(x = x, y = y), fill = NA, color = "forestgreen", size = 0.3) +

geom_sf(data = st_as_sf(buffered_area), fill = NA, color = "black", linetype = "dashed", size = 0.6) +

geom_sf(data = st_as_sf(study_a), fill = NA, color = "black", size = 0.9)+

coord_sf() +

ggtitle(paste0("Least cost path connectivity of large ", feature, "-areas in ",area)) +

theme_minimal()

g1

#save files to directory

feature

getwd()

#create new folder

{

setwd("path to folder")

date_str <- format(Sys.Date(), "%Y%m%d")

folder_name <- paste0(area,"_", feature, "_", date_str)

dir.create(folder_name, showWarnings = FALSE)

setwd(folder_name)

plot(patchyCost)

#save all shp and tif

getwd()

date_str <- format(Sys.Date(), "%Y%m%d")

ggsave(

filename = paste0(feature,"_", area, "_", date_str, ".png"),

plot = g1,

width = 11,

height = 8)

writeVector(buffered_area, paste0("buffered_area_", area, "_", date_str), filetype = "ESRI Shapefile", overwrite = TRUE)

writeVector(study_a, paste0("study_area_", area, "_", date_str), filetype = "ESRI Shapefile", overwrite = TRUE)

writeRaster(merged_patches, paste0("study_area_", area, "_", date_str, ".tif"), filetype = "GTiff", overwrite = TRUE)

crs(expanded_r) <- "EPSG:3035"

writeRaster(expanded_r, paste0("corridor_large_", area, "_", date_str, ".tif"), filetype = "GTiff", overwrite = TRUE)

r <- rast(mpgPlot_df)

writeRaster(r, paste0("corridor_original_", area, "_", date_str, ".tif"), filetype = "GTiff", overwrite = TRUE)

writeRaster(patchyCost, paste0("resistance_", area, "_", date_str, ".tif"), filetype = "GTiff", overwrite = TRUE)

str(merged_patches)

setwd("C:/Users/suanjak/Documents/Projekte/EU4Green/Connectivity")

getwd()

}